HIN 2018 PbPb run

FW needs

1- Jet algorithm and background subtraction

The background subtraction algorithms are still under studies and the internal deadline of end of end-of-April will be applied to select among the various options.

We are currently studying the performances of the “Chunky donut” and of the “phi-ring” background subtraction algorithms. The background subtraction needs to be performed prior to the jet finding. The phi-ring subtraction algorithm divides the towers eta-phi map in strips of one tower width (1-3 tower width being studied). For each strip the average energy value is calculated and it is subtracted from each tower of the strip. A saturated tower will be treated as any tower in the strip.

Whatever the final choice of algorithm will be, the phi strips at |ieta| = 28 should be masked.

2- EG

The EG algorithms used for pp collision will be used also for Pb-Pb collsions. Some changes at the SW level will be require to add more flexibility on the EG triggered event selection.  Variable such as FG bit, H/E cuts, shape cut should be programmed with different settings. Changes in the configuration code are foreseen and already implemented by the HI crew in private CMSSW version. Integration in the main L1 CMSSW releases should be organized.

3- Muons

No FW changes are foreseen for the Muon algorithms

4- Minimum Bias (MB) trigger

The MB trigger is based on the HF Fine Grain (FG) bits. The algorithm foresees a trigger when one of the HF tower on at least one side of HF (OR) or one tower on each side (AND) is above a defined ADC threshold. This is already implemented in the current FW and does not need to be changed. We call this trigger MB1

5- Minimum Bias Ultra Peripheral Collision (MB UPC) trigger

For this trigger, we need to duplicate the MB trigger to have a second threshold available. Indeed, to select MB UPC events, we need to trigger events when the leading tower in HF is bellow a defined ADC threshold. This insures selecting non-hadronic collisions. To reach this goal, we need a second threshold (MB2) and we will trigger on the NOT MB2. This requires FW changes at HF, layer-1, layer-2 and uGT level. The algorithm implementation is the same of MB1 but using a different ADC threshold (higher threshold) for the towers.

6- High Multiplicity Ultra Peripheral Collision (HM UPC) trigger

This trigger is based on the imbalanced of ETT in the barrel calorimeter between the positive and negative side. The selection will use an imbalanced variable defined as:

Imb(ETT) = (ETT_pos – ETT_neg)/ETT_tot,

where ETT_pos and ETT_neg are the ETT in the positive and negative side of the calorimeter respectively. ETT_tot is the sum of ETT_pos and ETT_neg.

Having access to ETT_pos/neg level information at uGT level would allow to implement the trigger. The data format should be the same as all the other Et sums.

7- Centrality trigger

The centrality trigger is based on HF Et sums and is the same as the one used in 2015. To select events from different centrality classes (degree of overlap between colliding ions) we rely on the energy deposited in HF. We foresee 5 centrality classes at the moment. The logic of the trigger as implemented in 2015 was the following:

- For each event, compute the HF Et sums

- A look up table (LUT) must be available, it will contains the bounds of each centrality classes expressed in HF Et values

- The LUT should contain 10 values (1 min. and max. per centrality class)

- The algorithm should define using the LUT in which class an event belongs to

The centrality trigger should spit out a 5 bits word (one bit per centrality class)

The bounds of the different centrality classes can overlap to improve the sharpness of the efficiency turn on curve. Therefore, it should be possible for an event to trigger more than one centrality class.